VIPX.386 Description, Limitations and Configuration Document

Current Version:        1.17

Date:                   931118

Table of Contents

-----------------

I.   Description

II.  Limitations

A.  Windows 3.X Versions

B.  Version Compatibility with Dedicated IPX

C.  Packet Size Limitations

D.  Memory Manager Limitations

III. System Settings in the SYSTEM.INI File

A.  [386Enh] Section

1.  TimerCriticalSection

B.  [VIPX] Section

1.  VipxMappingPages

2.  VipxFailOverSizedPackets

3.  VpicdFix

4.  AutoIrqVirtualize

5.  VirtualizeIrq[0-F]

6.  VipxErrorMessages

7.  VipxWarningMessages

8.  VipxOutDebugStrOnErrors

9.  VipxOutDebugStrOnWarnings

10. VipxBreakOnErrors

11. VipxBreakOnWarnings

IV.  Programming to the VIPX Interface

A.  Getting the VIPX API Entry Point

B.  Memory Management Issues

C.  APIs Specific to VIPX.386

D.  Handling Reentrancy Issues

E.  Using NWIPXSPX.DLL

V.   Global DOS TSRs using IPX/SPX

I.  Description

---------------

VIPX.386 is a Windows 3.X virutalization driver for IPXODI.COM

driver.  It virtualizes requests to the globally loaded IPX driver.

When a request is made to IPX, VIPX will allocate a request buffer in

the system's global memory, copy the original request to the global

buffer and give the global request to IPX.  When the global request

completes, IPX will call VIPX.  VIPX will then copy any results back

to the original request buffer and call the application which

submitted the request.

II.  Limitations

----------------

II.A.  Windows Versions

-----------------------

VIPX supports Windows versions 3.0, 3.0a and 3.1.  However, there is

some functionality which VIPX will only support in Windows 3.1. This

is LAN IRQ virtualization for avoiding deadlocks.  Under WIndows 3.0

and 3.0a, VIPX will not virtualize LAN IRQs.  See section III.B.5

VirtualizeIrq[0-F].

A patch has been developed by Microsoft for Windows 3.1.

The patch is VTDA.386 found in WW0863.EXE on MicroSoft's BBS.  The

BBS phone number is (206)936-6735.  Microsoft found a bug in the

their Virtual Timer Device (VTD) driver which may cause a deadlock

with the NetWare shell (NETX).

The deadlock occurs as a result of the shell waiting for the timer

tick to advance.  When an NCP is sent, the shell will start it's

retry timeout countdown.  This is essentially a loop waiting for the

timer tick value to advance to a certain point.  The deadlock occurs

when the tick value never increments and thus the shell is left in an

infinite loop.  The reason the tick count is never incremented is

that the timer interrupt is not allowed to be simulated into V86 mode

under certain circumstances.  The patch to the VTD allows the timer

interrupts to be simulated.

II.B.  Version Compatibility with Dedicated IPX

-----------------------------------------------

Novell offically ceased maintenence on the dedicate IPX driver

(IPX.OBJ) verion 3.10 dated 11-21-91.  The last version of VIPX to

explicitly support that driver is 1.10.  The current version of VIPX

supports IPXODI.COM only.  It is strongly recommended that you update

your dedicated IPX driver with IPXODI.COM when using versions of VIPX

later than 1.10.

II.C.  Packet Size Limitations

------------------------------

VIPX.386 will only virtualize packets which are 8000 (decimal) bytes

or less.  Any DOS and Windows IPX or SPX applications which use

networks with a physical packet size greater than 8000 bytes may not

work with VIPX.386.  For example, IBM Token Ring can be configured to

run with 16K packets.  A request by a DOS or Windows IPX application

to send a 16K packet will be truncated to 8000 bytes.  On the other

hand, any 16K packet that is received by the LAN driver will be

dropped because VIPX cannot allocate a packet big enough to handle

it.

II.D.  Memory Manager Limitations

---------------------------------

When an request is passed up from IPX, VIPX will immediately test the

request buffer to see if it is in global memory already.  If it is in

global memory, the request will be passed back down to IPX without

any virtualization.  For example, a TSR loaded before Windows is

considered to be in global memory.  If that TSR calls IPX, VIPX will

test the requests and pass them back down to IPX.  This is done

because there is no need to virtualize requests which are already

global.

The use of UMB memory complicates the test for global memory.  There

are two basic senarios.  Senario One is when a TSR has been loaded

high before Windows was loaded.  In this case, IPX requests will come

from global UMB memory.  VIPX will simply pass these requests back

down to IPX.  Senario Two is when a TSR is loaded high in a Windows

DOSBOX.  In this case, IPX requests will come from local UMB memory.

VIPX will virtualize these requests.

Using HIMEM.SYS and EMM386.SYS, all tests for global UMB memory will

work properly under both of senarios.

However, there are some memory managers that do not work properly

under Windows.  With these drivers, all LOCAL DOSBOX UMBs look as if

they are GLOBAL UMBs.  Under Senario Two, when a TSR calls IPX, VIPX

will test the request buffer and think that it is in global UMB

memory when it is really in LOCAL UMB memory.  As a result, VIPX will

pass a local ECB to IPX without virtualization.  The normal result of

this is a hung machine or data corruption.

III.  System Settings in the SYSTEM.INI File

--------------------------------------------

III.A.  [386Enh] Section

------------------------

III.A.1.  TimerCriticalSection

------------------------------

As of version 1.15 of VIPX, TimerCriticalSection is required to be set

on.  The recommended setting is as follows:

[386Enh]

TimerCriticalSection=10000

The reason for this parameter is to avoid a deadlock with the LAN IRQ

Virtualization code.  See section III.B.5 VirtualizeIrq[0-F].

III.B.  [VIPX] Section

------------------------

Under most circumstances, VIPX will work fine under the default

configuration.  However, there may be some applications which require

custom configuration of the driver.  This is a list of SYSTEM.INI

parameters which can be used to configure VIPX:

[VIPX]

VipxMappingPages                =[number of 4K pages]   (default = 16)

VipxFailOverSizedPackets        =[ON|OFF|TRUE|FALSE]    (default = OFF)

VpicdFix                        =[ON|OFF|TRUE|FALSE]    (default = ON)

AutoIrqVirtualize               =[ON|OFF|TRUE|FALSE]    (default = ON)

VirtualizeIrq[0-F]              =[ON|OFF|TRUE|FALSE]    (default = OFF)

(BELOW ARE CONFIGURATION PARAMETERS FOR BETA VERSIONS ONLY!)

VipxErrorMessages               =[ON|OFF|TRUE|FALSE]    (default = OFF)

VipxWarningMessages             =[ON|OFF|TRUE|FALSE]    (default = OFF)

VipxBreakOnErrors               =[ON|OFF|TRUE|FALSE]    (default = OFF)

VipxBreakOnWarnings             =[ON|OFF|TRUE|FALSE]    (default = OFF)

VipxOutDebugStrOnErrors         =[ON|OFF|TRUE|FALSE]    (default = OFF)

VipxOutDebugStrOnWarnings       =[ON|OFF|TRUE|FALSE]    (default = OFF)

III.B.1.  VipxMappingPages

--------------------------

This is the number of pages that VIPX can use to globalize requests

to the global IPXODI.COM driver.  VIPX is not absolutely guaranteed

to have all of these pages available at any one point, since this is

the requested number of pages for SHARED global mapping which VIPX

makes to the Windows VMM at initialization time.

III.B.2.  VipxFailOverSizedPackets

----------------------------------

This parameter tells VIPX to fail any requests which require more

than the maximum allowed globalization size.  The actual maximum will

vary according to which media the user is on.  The absolute maximum

is 8000 (decimal) bytes.  With media that have smaller packets than

8000 bytes, the maximum allowed size is the maximum packet size that

can be put onto the media.

III.B.3.  VpicdFix

------------------

III.B.4.  AutoIrqVirtualize

---------------------------

VpicdFix and AutoIrqVirtualize are synonyms for the same option.

The VPICD VxD has a problem of allowing an interrupt to be simulated

to a VM which previously held the Windows Critical Section but currently

does not.  (For more details, see III.B.5. VirtualizeIrq[0-F]).  This

causes a deadlock situation when LAN I/O is attemped in another VM.

To get around the problem, VIPX will take over the job of virtualizing

LAN IRQs and will simulate LAN interrupts only to the owner of the

Windows Critical Section.

This option is used to turn on or off the automatic detection and

virtualization of LAN IRQs when ODI LAN drivers or dedicated LAN

drivers are loaded.  The reason for this option is to allow OEMs to

virtualize their LAN drivers with their own VxDs.

III.B.5.  VirtualizeIrq[0-F]

----------------------------

This is option is automatically configured excepted on machines using

the IBM LAN Support Program.  This parameter tells VIPX to virtualize

the specified (hex) Interrupt ReQuest (IRQ) line.  This feature is

mainly designed to assist the IBM LAN Support Program in avoiding the

"Black Screen of Death".

Because of a design flaw in the VPICD.386 driver, network interrupts

will sometimes deadlock a PC running Windows 3.X.  VIPX 1.15 and later

attempts to avoid the deadlock by virtualizing the Network Interface

Card's (NIC) IRQ(s).  With ODI and dedicated IPX (IPX.OBJ) drivers,

VIPX will automatically read the configuration of the NIC from the

driver and virtualize the selected IRQs.

The only time that VIPX cannot automatically detect the LAN IRQs is

when using the IBM LAN Support Program with either SLANSUP.OBJ or

LANSUP.COM.  In this case, the LAN IRQ is not readable from the

driver.  The reason (so I am told) is that IBM LAN Support does not

have an API to read the IRQ setting on the NIC:  The only way to get

this information is to read the NIC hardware itself.  The problem

with doing this is that the hardware can be Token Ring, PCN2 or

Ethernet.  This means that VIPX would have to be aware of all of the

different configurations of all of these hardwares. Instead of this,

VIPX requires the IBM LAN Support user to specify the NIC IRQ in the

[VIPX] section of the SYSTEM.INI.  IRQs range from 0 to F (hex).  An

example is listed below:

[VIPX]

VirtualizeIrq2=TRUE

VirtualizeIrq3=TRUE

In this example, VIPX will virtualize both IRQ 2 and IRQ 3.  VIPX can

virtualize upto 4 different LAN IRQs.  The reason for virtualizing

multiple IRQs is to allow other LAN cards and protocols to be

installed on the same PC and prevent them from deadlocking the

machine.  For example, you may have IPX running through an NE2000

card on IRQ 3 and TCP/IP running through to an IBM Token Ring card on

IRQ 2.

One side note.  On an AT class PC, IRQ 2 is is really IRQ 9 on the

physical machine.  IRQ 2 is a special cascade interrupt used to

access IRQs 9-F.  Whenever a NIC raises the IRQ 2 line, the hardware

configuration on the PC's bus actually issues an IRQ 9.  Also, if a

NIC is configured to IRQ 2, the LAN driver during its initialization

procedure will detect that the PC is an AT class machine and will

correctly configure its software to use IRQ 9.

Getting to the point, the Windows VPICD VxD will not allow IRQ 2 to

be exclusively virtualized by any VxD.  However, when VIPX detects

that IRQ 2 is to be virtualized, it changes the IRQ to 9.  So if you

are using the IBM LAN Support Program on an IBM Token Ring NIC set to

IRQ 2, you should have not trouble.  The following two configurations

are equivalent:

[VIPX]

VirtualizeIrq2=TRUE

or

[VIPX]

VirtualizeIrq9=TRUE

One important note when using VIPX 1.15 or higher:  Set the

TimerCriticalSection under the [386Enh] section in the SYSTEM.INI as

follows:

[386Enh]

TimerCriticalSection=10000

The reason for this is that the timer interrupt can cause a deadlock

with the virtualization of the the LAN interrupt.

You should note that the VirtualizeIrq[0-F] can be used to override

any auto-virtualization of LAN IRQs.  For example, if your ODI LAN

driver is configured for IRQ 5, VIPX will automatically detect that

IRQ 5 is to be virtualized.  However, if you have

[VIPX]

VirtualizeIrq5=FALSE

in your SYSTEM.INI, VIPX will override the automatic detection of IRQ

5 and not virtualize the interrupt.

Another way to disable the auto-virtualization feature of VIPX is

with the VpicdFix or AutoIrqVirtualize parameters (See III.B.3.

VpicdFix or III.B.4 AutoIrqVirtualize).

III.B.6.  VipxErrorMessages

---------------------------

III.B.7.  VipxWarningMessages

-----------------------------

III.B.8.  VipxOutDebugStrOnErrors

---------------------------------

III.B.9.  VipxOutDebugStrOnWarnings

-----------------------------------

III.B.10. VipxBreakOnErrors

---------------------------

III.B.11. VipxBreakOnWarnings

-----------------------------

These parameters are only enabled in Beta versions of VIPX. With Beta

software, the user can enable/disable these parameters to make VIPX

print error/warning messages to the user screen and debugger screen.

They can also instruct VIPX to execute a break point whenever an

error/warning occurs so that debugging can be done on site.

IV.  Programming to the VIPX Interface

--------------------------------------

IV.A.  Getting the VIPX API Entry Point

---------------------------------------

VIPX supports API calls from 16-bit DOS and Windows applications.

This is the interface which NWIPXSPX.DLL uses to issue calls to VIPX.

To get the VIPX Device V86/PM API Entry Point, a 16-Bit DOS/Windows

application must do the following:

DWORD VIPX_API_ADDR;

#define WIN_GET_DEV_API_ENTRY_PT 0x1684

#define VIPX_DEVICE_ID 0x0200

GetVipxApiAddr ()

{

_asm

{

mov     ax, WIN_GET_DEV_API_ENTRY_PT

mov     bx, VIPX_DEVICE_ID

int     2fh

mov     word ptr VIPX_API_ADDR+2, ES

mov     word ptr VIPX_API_ADDR, DI

}

if (!VIPX_API_ADDR)

{

print error - NO PM API! RIGHT VERS OF VIPX?

handle error

}

}

Once the API address is obtained, an application then can make a far

call to the VIPX API Handler to do two primary functions:

1.  Request an 16-Bit PM IPX/SPX operation (SendPacket, etc...).

2.  Request a device specific operation (GetVersion, GetDiagnotics).

The way that VIPX differentiates between the two types of APIs is by

the value in the BH register.  If BH equals 00h, then VIPX will

handle a 16-Bit Protected Mode IPX/SPX operation.  NOTE:  The V86

IPX/SPX API interface (i.e. DOS application) is done through the

globally loaded IPX TSR itself.  This way, DOS applications do not

need to change the way they access the IPX/SPX APIs under Windows and

DOS.

If BH equals the character 'V' (56h), then VIPX will will perform a

V86/PM Device Specific API.  These APIs are listed below (and will be

presented in full specification later):

GetVipxVersion

GetVipxDiagnostics

For IPX/SPX operations (BH = 00h), a 16-bit Windows program must

follow the IPX/SPX API specification in the NetWare System Calls-DOS

manual under the Communications chapter.  Coverage of the DOS IPX/SPX

APIs is beyond the scope of this document.  However, an example is

provided below for reference:

DWORD VIPX_API_ADDR;

int ccode;

CallPmIpxSpxApi ()

{

_asm

{

mov     bx, (IPX operation number)

(set other registers as specified by the function)

call    VIPX_API_ADDR

(save return registers as specified by the function)

ErrorCondition = (function error condition)

}

if (ErrorCondition)

{

print error - function failed!

handle error

}

}

IV.B.  Memory Management Issues

-------------------------------

One problem with using the IPX/SPX interface in Windows applications

is that all memory used by VIPX must be page-locked.  Since

VIPX is an asynchronous driver, all application ECBs, fragments and

ESRs must be in locked memory to insure that they can be accessed

during interrupt time.  If an ECB, fragment or ESR is in moveable

memory, it could move when VIPX "owns" that memory.  For example,

assume an ECB is located at 16-bit PM address 0A10:0000 in moveable

memory.  Assume this address tranlates to 32-bit linear address

8140ACB0.  Assume also that the application gives the ECB to VIPX and

VIPX commences to work on it.  Now if Windows does garbage

collection, the linear mapping for the 16-bit address 0A10:0000 may

change from 8140ABC0 to 81408100.  When VIPX tries to access the old

linear address 8140ABC0, memory corruption will occur because VIPX is

accessing the wrong memory.

To avoid memory corruption, follow the guidelines below (your

comments on the effectiveness of these guidelines are appreciated):

1.  Read Chapter 15 (Memory Management) and 16 (More Memory

Management) of the MS Windows SDK Version 3.1.  These will explain

many terms and concepts which will be discussed below.

2.  Define all ESRs, routines called by ESRs and data accessed by

ESRs in one module.  In the .DEF file for your project, define the

module's code and data segments as FIXED.

3.  Use GlobalPageLock() on all dynamically allocated heap memory

(via GlobalAlloc() or LocalAlloc()) which is to be used by VIPX or an

ESR.  GlobalPageLock() informs the GUI kernel memory manager to fix

the linear base of the selector until a GlobalPageUnlock() is done on

the memory.

4.  When using GlobalAlloc(), use the GMEM_DDESHARE and GMEM_FIXED

options.  GMEM_DDESHARE will allocate the memory (so I am told) high

up on the global heap.  This will avoid using conventional memory and

will allow room for other Windows applications to load after your

application has been loaded and initialized.  The GMEM_FIXED option

tells the GUI kernel memory manager that the memory is FIXED.  This

does not mean that the linear address will remain unchanged, but it

does mean that the segmented address (selector:offset) will not

change.  After the call to GlobalAlloc(), call GlobalLock() to

derefernce the global memory handle.  This provides a far pointer

(selector:offset) to the memory which will not change.  Next call

GlobalPageLock() to fix the linear base of the selector so that the

32-bit linear mapping of the selector:offset does not change.

5.  If local moveable or discardable code or data segments are used

for ECBs, fragments or ESRs, use GlobalPageLock() on those segments.

6.  After you load your application and then you get a message from

Windows that there are not enough resources to load another program,

try changing the attributes of your segments. I believe this

condition occurs because too much conventional memory has been

allocated to fixed memory objects and Windows needs to allocate

conventional memory to load a program.  One strategy may be to do a

GlobalCompact() with a argument of -1 at initialization time.  This

will move all moveable segments and discard all discardable

segments.  Next call GlobalAlloc() for one large moveable global

object which contains all of your ECBs, fragments and data which must

be accessed from VIPX or your ESRs.  Next call GlobalLock() to

dereference the global handle.  Last call GlobalPageLock() to fix the

linear address of the object.  (I have not tried this yet, so there

may be some problems with it).  Also make your ESR code as small as

possible using PostMessage to indicate to your application that an

ECB has completed.  This way, your FIXED code segment will have

little impact on conventional memory.

7.  Read Chapter 4 of The Windows Programmer's Guide to DLLs and

Memory Management by Mike Klein.

IV.C.  APIs Specific to VIPX.386

--------------------------------

There are two APIs via the VIPX API entry point which are specific

to VIPX.386 only.  These are:

GetVipxVersion

GetVipxDiagnostics

The device specific APIs are called in the same manner as the IPX/SPX

APIs except that BH = 56h.  Some sample code is listed below:

DWORD VIPX_API_ADDR;

#define VIPX_DEVICE_API_TAG 'V'

int ccode;

CallVipxDeviceSpecificApi ()

{

_asm

{

mov     bh, VIPX_DEVICE_API_TAG

mov     bl, api_function_number

(set other registers for API function)

call    VIPX_API_ADDR

mov     ccode, ax

}

if (!ccode)

{

print error - api function failed!

handle error

}

}

The full API specification for GetVipxVersion is as follows:

GetVIPXVersion

This procedure puts the VIPX major and minor version numbers into

the client AH and AL.

To call this API a DOSBOX or Windows 3.0 application

must first get the VIPX Device API entry point (see

VIPX_V86_API_Handler or VIPX_PM_Handler).  Then

it must do the following:

Example:

DWORD VIPX_API_ADDR;

BYTE VIPX_Major_Version;

BYTE VIPX_Minor_Version;

#define VIPX_DEVICE_API_TAG 'V'

#define VIPX_GET_VERSION 0x00

int ccode;

main ()

{

_asm

{

mov     bh, VIPX_DEVICE_API_TAG

mov     bl, VIPX_GET_VERSION

call    VIPX_API_ADDR

mov     ccode, ax

mov     VIPX_Major_Version, bh

mov     VIPX_Minor_Version, bl

}

if (!ccode)

{

print error - api function failed!

handle error

}

}

Entry:

BH      056h - VIPX Device API

BL      0 - Get VIPX Version function

Exit:

BH      VIPX Major Version

BL      VIPX Minor Version

AX      Return code

0000h - success

Uses:

AX, BX, Flags

The full API specification for GetVIPXDiagnostics is as follows:

GetVIPXDiagnostics

This procedure puts the diagnostics for VIPX into the

specified client diagnostic buffer.  The buffer is 112 bytes

and has the following structure:

VIPXDiagStruc   STRUC

MallocCount                     dd      ?

MallocErrorCount                dd      ?

FreeCount                       dd      ?

AllocPageCount                  dd      ?

FreePageCount                   dd      ?

PMRequestCount                  dd      ?

TSRServiceCount                 dd      ?

ServiceAESEventCount            dd      ?

ServiceIPXEventCount            dd      ?

PostEventCount                  dd      ?

IPXGetECBCount                  dd      ?

IPXGetECBBadPacketCount         dd      ?

IPXGetECBOutOfResource          dd      ?

IPXCountECBCount                dd      ?

IPXReturnECBCount               dd      ?

IPXRequestCount                 dd      ?

FreeECBCount                    dd      ?

AllocECBCount                   dd      ?

LockECBCount                    dd      ?

UnlockECBCount                  dd      ?

OpenSocketCount                 dd      ?

CloseSocketCount                dd      ?

SendPacketCount                 dd      ?

ListenCount                     dd      ?

IPXScheduleEventCount           dd      ?

AESScheduleEventCount           dd      ?

CancelECBCount                  dd      ?

PMInt2FCount                    dd      ?

AllocPageErrorCount             dd      ?

FreePageErrorCount              dd      ?

FreeECBErrorCount               dd      ?

MaxVipxAllocSize                dd      ?

MaxVipxFragsSize                dd      ?

TsrServiceReenterCnt            dd      ?

IPXGetEcbSktNotOpenCnt          dd      ?

IPXGetEcbNoEcbAvailCnt          dd      ?

ENDS

NOTE:  Starting with VIPX 1.16, MaxVipxAllocSize, MaxVipxFragsSize, and

TsrServiceReenterCnt have been updated to DWORDs.  Also IPXGetEcbSktNotOpenCnt

and IPXGetEcbNoEcbAvailCnt have been added.

To call this API a DOSBOX or Windows 3.0 application

must first get the VIPX Device API entry point (see

VIPX_V86_API_Handler or VIPX_PM_Handler).  Then

it must do the following:

Example:

DWORD VIPX_API_ADDR;

VIPXDiagStruc Diag;

#define VIPX_DEVICE_API_TAG 'V'

#define VIPX_GET_DIAGNOSTIC 0x01

int ccode;

main ()

{

_asm

{

mov     bh, VIPX_DEVICE_API_TAG

mov     bl, VIPX_GET_DIAGNOSTIC

mov     ax,ds

mov     es,ax

mov     di, offset Diag

mov     cx, size VIPXDiagStruc

call    VIPX_API_ADDR

mov     ccode, ax

}

if (!ccode)

{

print error - api function failed!

handle error

}

}

Entry:

BH      056h - VIPX Device API

BL      1 - Get VIPX Diagnostics

ES:SI   ptr to client VIPX Diagnostics buffer.

CX      Length of diagnostics buffer in bytes

Exit:

ES:SI   client's buffer now has VIPX diags.

AX      Return code

0000h - success

0FFFEh - diagnostics buffer too short

Uses:

AX, SI, ES, Flags

IV.D.  Handling Reentrancy Issues

---------------------------------

A Windows application which uses the IPX/SPX APIs via VIPX must

handle reentrancy issues.  Since the IPX/SPX API is interrupt driven,

any code running while interrupts are enabled can (and will) be

interrupted at any time by IPX/SPX.  When an IPX event completes, an

ESR for that event is called.  The ESR is entered with interrupts

disabled.  If that ESR reenables interrupts, then it can be

reentered.  This is usually a problem if:

1.  The application has limited stack space

2.  The application's ESR uses global variables

An ESR must protect itself from these two reentrancy issues if it

reenables interrupts.  Interrupts can be reenabled by several

different places.  One of the most notorious is the PostMessage()

Windows call.  Any ESR using PostMessage() must be reentrant.

Another notorious enabler of interrupts is IPXSendPacket.

IPXSendPacket will call the LSL to transmit a packet.  The LSL,

however,  will reenable interrupts during transmission of a packet.

This can cause reentrancy into any ESR that calls IPXSendPacket.

An ESR can handle reentrancy issues by

1.  Switching to a stack exclusively for the ESR to use.

2.  Provide a reetrancy queue.

The stack switching is usually done in assembly.  It is by far the

easiest and most efficient way to protect your stack.  The code for

stack switching is listed below:

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

memL equ 1 ; large model

?PLM = 1 ; pascal conventions

?WIN = 1 ; epilogue/prolog code

INCLUDE cmacros.inc

sBegin  DATA

assumes DS, DATA

ReenterCnt      db      0

OldSs           dw      0

OldSp           dw      0

EsrStack        db      4096 dup (0)

EndEsrStack label byte

sEnd    DATA

sBegin  CODE

assumes CS, CODE

;

;       This is the front-end procedure to the C function "ReceiveESR"

;

externFP cESR

PUBLIC aESR

aESR    proc FAR

nop

nop

nop

nop

; Don't know why Windows needs to inc bp

inc     bp

push    bp

mov     bp, sp

; Save data segment

push    ds

; Fix to our data segmented

mov     ax, DGROUP

mov     ds, ax

; Update our reenter cnt

inc     ReenterCnt

; Do we need to switch stack to ESR stack?

cmp     ReenterCnt, 2           ;are we reentering?

jae     CallEsr                 ;y: call esr

mov     ax, ss

cmp     ax, DGROUP              ;same stack seg?

jne     SwitchStack             ;n: switch

lea     ax, EsrStack            ;ax=offset of EsrStack

cmp     sp, ax                  ;below the esr stack?

jb      SwitchStack             ;y: switch

lea     ax, EndEsrStack         ;ax=offset of end of EsrStack

cmp     sp, ax                  ;above the esr stack

jae     SwitchStack             ;y: switch

; Call the esr

CallEsr:

push    es

push    si

cCall   cESR

lea     sp, [bp-2]

; Update reenter cnt

dec     ReenterCnt

; Switch stacks back?

jz      SwitchStackBack

; return to IPX

EsrExit:

; restore ds and bp

pop     ds

pop     bp

dec     bp

ret

SwitchStack:

mov     ax, sp

mov     OldSp, ax

mov     ax, ss

mov     OldSs, ax

lea     ax, EndEsrStack

mov     sp, ax

mov     ax, ds

mov     ss, ax

jmp     CallEsr

SwitchStackBack:

mov     ax, OldSp

mov     sp, ax

mov     ax, OldSs

mov     ss, ax

jmp     EsrExit

aESR    endp

sEnd    CODE

end

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The above code switched to a 4K stack before calling the main ESR

code.  Notice that there is a reentrancy count to determine if a stack

switch should occur.  This is used to prevent switching the stack

again if the ESR is reentered.

The main ESR code is written in C.  An example is listed below:

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

void far pascal cESR(ECB far *esr_ecb)

{

static ECB FAR * EcbQueue;

static ECB FAR * EcbQueueTail = (ECB FAR *)&EcbQueue;

static WORD ReenterFlag = 0;

// Check for reentrancy

if (ReenterFlag)

{

// Enqueue the ECB

EcbQueueTail->Link = (void far *)esr_ecb;

EcbQueueTail = (ECB far *)esr_ecb;

esr_ecb->Link = NULL;

return;

}

// Set the reentrany flag to indicate that we can't allow

// another thread through the ESR.

ReenterFlag++;

// Process the current ECB

NextEcb:

// Do some work here if need be -- BUT BE QUICK!

...

// Post a message to the Windows App that the event happened

PostMessage (

hMainWnd,

USER_DEFINED_IPX_EVENT_HAPPPENED,

0,

(DWORD) esr_ecb);

// Check for anything on the queue

CheckQueue:

if (EcbQueue)

{

esr_ecb = EcbQueue;

EcbQueue = (ECB FAR *)(EcbQueue->Link);

if (!EcbQueue)

EcbQueueTail = (ECB FAR *)&EcbQueue;

goto NextEcb;

}

// Reset the reentrancy flag to indicate that the

// first thread is done

ReenterFlag--;

}

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

One side note about the code above.  PostMessage() will queue a

message onto an applications message queue.  This queue is limited in

size.  If the application is fairly LAN I/O intensive, the message

queue can be overrun.  In this case, PostMessage() will simply drop

the message.  If this happens, your application will never get

notified of the event.  To insure that messages are never dropped,

you may want to increase the size of your message queue.  Another

alternative is to have the message indicate that an event happened

and the message loop will then poll all ECBs to determine which ones

completed.  This way, if a message is dropped, it won't matter:  the

main message loop will still detect that the ECB completed and will

be able to reuse the ECB.

IV.E.  Using NWIPXSPX.DLL

-------------------------

Most Windows applications will use the IPX/SPX APIs via the

NWIPXSPX.DLL.  The reason is that NWIPXSPX.DLL address the issues for

both Standard and Enhanced mode Windows.  Under Standard mode,

NWIPXSPX will use TBMI2.COM to virtualize the local ECBs passed to

it.  Under Enhanced mode, NWIPXSPX.DLL will use VIPX.386 to

virtualize ECBs.  If your Windows application is targeted toward

Enhanced mode only, then you may find it more convenient to program

directly to the VIPX interface in assembly.  However, if you are

interested in using an established C interface for IPX/SPX, then

NWIPXSPX.DLL is the way to go.  There is example code using NWIPXSPX.DLL

on the Compuserve NOVDEV forum.

V.  Global DOS TSRs using IPX/SPX

---------------------------------

TSRs loaded before Windows can call IPX without the intervention of

VIPX.  This is done by ORing the BX register with 8000h (i.e. setting

the high-bit of the function number).  This is a previously

undocumented feature in IPX to support the globally loaded NETX

driver.  When IPX sees that a request has the high-bit set, it will

assume that the request came from a globally loaded TSR and will not

forward the request to VIPX.

The only problem with the high-bit is that SPX does not recognize

it.  When SPX gets a request with a high-bit, SPX ignores it and

reposts the ECB to IPX without the preserving the high-bit.  VIPX

will then get the IPX request and then determine if the ECB is really

loaded in global memory.  If so, then it will simply pass the global

request back down to IPX with the high-bit set.