TN210_20.DOC

MULTI-PORT OPERATION

************************

Multiple  TNCs and radios can be interconnected to form a "node stack".   A

diode  matrix is used to route the signals in proper sequence between the TNCs.

Interconnecting  more than two TNCs does decrease the thruput since the circuit

is no  longer operating full duplex.  Diode matrix node stacks of up to 14 TNCs

are in  operation.   Shown  is a four TNC matrix circuit.  For more  than  four

TNCs,  the  circuit  is  appropriately  expanded.

RS-232 Four Matrix

Note:   On the DB-25 connectors, all pin 1's are connected together as  are

all pin 7's.  On EACH DB-25, pins 10 and 23 are jumpered together as are pins 4

and 20.  Comment:  The original TAPR TNC-2 circuit board layout  mistakenly had

CTS connected to pin 20, instead of pin 4.  It took awhile before manufacturers

licensed by TAPR caught and corrected this error.  Meanwhile, it is good policy

to  simply jumper 4 and 20 in case  of a mix of older and newer TNCs being used

in the stack.

|-->|-- C2-2                       |-->|-- C2-5

Connector 1-3----|-->|-- C3-2     Connector 1-20----|-->|-- C3-5

|-->|-- C4-2                       |-->|-- C4-5

|-->|-- C1-2                       |-->|-- C1-5

Connector 2-3----|-->|-- C3-2     Connector 2-20----|-->|-- C3-5

|-->|-- C4-2                       |-->|-- C4-5

|-->|-- C1-2                       |-->|-- C1-5

Connector 3-3----|-->|-- C2-2     Connector 3-20----|-->|-- C2-5

|-->|-- C4-2                       |-->|-- C4-5

|-->|-- C1-2                       |-->|-- C1-5

Connector 4-3----|-->|-- C2-2     Connector 4-20----|-->|-- C2-5

|-->|-- C3-2                       |-->|-- C3-5

This  circuit requires 24 small signal diodes, such as 1N914's, four  DB-25

male  connectors, a supply of small various color coded wire and a small  piece

of perf  board  on  which to mount the diodes and interconnecting  wires.   The

circuits  are wired in sequence.  For Connector 4, for instance, pin 3 is wired

to the anode side of three diodes in parallel.  The cathodes go to Connector 1,

pin 2,  Connector  2,  pin  2 and connector 3, pin 2.  Connector 4  pin  20  is

connected  to  the  anode  of three diodes in parallel.   The  cathodes  go  to

Connector 1, pin 5, Connector 2, pin 5 and connector 3, pin 5.  The wire bundle

going  from each of the DB-25's to the perf board can be any length you prefer,

but possibly 16 inches would be adequate.

After  the  circuit is wired, use an ohm meter to check continuity on  each

circuit.   As  in the dual port configuration, make sure all TNC's are set  for

9600  baud on their RS-232 ports.  Following power up it will take from 6 to 10

seconds for the adjacent node broadcasts to "wake" them up.

For information on a commercially available 6-port diode matrix board, send

a SASE to: NorthEast Digital Assn, PO Box 563, Manchester, NH  03105-0563.

Provided on the distribution diskette is a binary ROM image of TheNet Plus.

Also  provided is the  SET210 utility which easily allows setting up the node's

callsign, SSID, parameters and password to appropriate values.

For those  considering the purchase of an EPROM programmer, JDR Electronics has

a suitable  model at low cost, as does Needham.    But from whatever source, be

sure to specify one that will handle CMOS chips.  Inexpensive EPROM erasers are

available from a number of dealers.

In  keeping  with  the  high standards set by NORD><LINK, the production of

TheNet Plus is  placed in the public domain with the understanding it is NOT to

be used commercially.

73 and enjoy!

Bill Beech, NJ7P

Jack Taylor, N7OO