VIPER VLB Installation/Utilities/Drivers Diskette v1.02

Release Notes

Release 1.02 of the Viper VLB Installation/Utilities/Drivers Diskette

includes changes to a number of items, including the Windows 3.1 drivers,

the VPRMODE.EXE utility program, and the GO.EXE installation program.  These

changes include:

Windows 3.1

1.  The problem with MS Access has been corrected.

2.  The "flashing" cursor seen in some low resolution modes has been fixed.

3.  Performance of the Windows drivers have been improved in a number of

areas, including text handling and circle drawing.

VPRMODE.EXE

1.  Several new monitor types have been added to the Monitor Menu.

2.  The default memory base address for the Viper VLB has been changed

from A0000000 to 80000000.  This was done to offer greater compatibility

with VESA Local Bus motherboards.  The A0000000 address remains one of

the six options for the memory base address.

3.  The VESA BIOS Extentions (VPRMODE VESA) have been changed to eliminate

lockup problems with some applications (CSHOW, FRACTINT, VPIC).

Additionally, the palette roll feature in VPIC is now supported in

resolutions below 1152x960.

GO.EXE

1.  GO.EXE will now automatically run the VPRMODE program during first-time

installations, eliminating potential software initialization problems.

2.  The TurboDLD Deluxe drivers are now included on the

Installation/Utilities/Drivers diskette, eliminating the need for a

separate ACAD driver disk.

***************************************************************************

** IMPORTANT VIPER VLB SOFTWARE INSTALLATION NOTES **

1.      DO NOT INSTALL THE VIPER VLB WINDOWS 3.1 DRIVERS INTO YOUR WINDOWS

DIRECTORY. DUPLICATE FILES NAMES EXIST!!

2.      It is best to use the VPRMODE.EXE utility to properly configure

the Viper VLB for your monitor BEFORE using Windows 3.1 or AutoCAD.

!!!  BEFORE USING THE VIPER VLB WINDOWS 3.1 OR AUTOCAD DRIVERS, USE  !!!

!!!  GO.EXE TO INSTALL THE VIPER VLB UTILITIES, AND USE VPRMODE.EXE  !!!

!!!  TO PROPERLY CONFIGURE THE VIPER VLB FOR YOUR MONITOR            !!!

** IMPORTANT WINDOWS 3.1 DRIVER INSTALLATION/CONFIGURATION NOTES **

1.      When the Windows 3.1 drivers are copied to your hard drive, a

driver setup & configuration utility, INSTALL.EXE, is also copied.

INSTALL.EXE should be used from within Windows to setup Windows

for the Viper VLB.

2.      Insure that Windows is configured for standard VGA mode before

using INSTALL.EXE. This can be done with the SETUP.EXE program found

in your WINDOWS subdirectory.

3.      INSTALL.EXE should be started by using the File|Run command in the

Windows Program Manager (File|Run  C:\VIPER\WIN31\INSTALL).

4.      After running INSTALL.EXE from within Windows, a Diamond Viper Setup

icon will be placed into the Windows Control Panel.  The Diamond

Viper Setup program can be used to change resolutions, colors, fonts,

and palette type.

UTILITY FILES

VPRMODE.EXE     command line and menu driven mode and monitor setting

utility.  Use VPRMODE.EXE to configure the Viper VLB

for your particular monitor, to change the VIPER VLB

memory address, to select a text display mode, and to

run Viper VLB diagnostics.

VPRMODE saves monitor timing information to the

VIPER.INI file.

NOTES:  1. use VPRMODE ? for command line option list.

2. the VPRMODE monitor configuration save screen

may be displayed after running some of the

VPRMODE commands.  If this happens, select

ENTER to save. The screen will not appear again.

3. KNOW YOUR MONITOR! Please refer to your monitor

manual or manufacturer for correct monitor timing

information before running VPRMODE, and before

contacting Diamond for technical assistance. If

you do not have this information, DIAMOND WILL

NOT BE ABLE TO HELP YOU with VPRMODE problems.

VIP.EXE         utility which will display or write to a file important

system configuration information which can be used as

an aid in trouble-shooting. Type VIP to display to screen,

and VIP <filename> to save to a file.

NOTES:  1. the VPRMODE save configuration screen may be

displayed after running VIP.  If this happens,

select Enter to save.  The screen will not

appear again.

VDIAG.EXE       diagnotic utility which will display color test patterns

in each resolution supported.

NOTES:  1. to test extended modes, load the VESA BIOS

extentions before running VDIAG.  This can

be done by typing  VPRMODE VESA at the command

prompt.

CHKPATH.EXE     utility which checks the PATH environment variable for the

specified string.  Is used in the AUTOEXEC.BAT file with

the VIPERPATH statement.

VIPER.INI       this file is used to store the monitor and configuration

information for the Viper VLB, and is updated each

time you choose to Save Configuration from the VPRMODE.EXE

program.  Drivers find the location of the VIPER.INI file

by checking the VIPERPATH environment variable.

VIPERPATH       an environment variable which is used by device drivers

(Windows 3.1, AutoCAD) to locate the VIPER.INI file.

This variable is set by the DOS SET command, and is

automatically appended to your AUTOEXEC.BAT file during

Viper VLB Utility installation.

!!!!!  IMPORTANT !!!!!

THE VIPER.INI FILE AND THE VIPERPATH STATEMENT ARE

NEEDED FOR PROPER USE OF THE VIPER VLB WINDOWS AND

AUTOCAD DRIVERS!  DO NOT REMOVE THEM FROM YOUR SYSTEM.